USE POLICY FOR PORTABLE OUTDOOR FIREPLACES, OUTDOOR FIREPLACES AND RECREATIONAL FIRES WITHIN THE CITY OF FORT SCOTT

DEFINITIONS (as written in 2012 International Fire Code)

PORTABLE OUTDOOR FIREPLACE:  A portable, outdoor, solid-fuel-burning fireplace that may be constructed of steel, concrete, clay, or other noncombustible material. A portable outdoor fireplace may be open in design, or may be equipped with a small hearth opening and a short chimney or chimney opening in the top.  2012 IFC, 202.

RECREATIONAL FIRE:  An outdoor fire burning materials other than rubbish where the fuel being burned is not contained in an incinerator, outdoor fireplace, portable outdoor fireplace, barbeque grill or barbeque pit and has a total fuel area of 3 feet or less in diameter and 2 feet or less in height for pleasure, religious, ceremonial, cooking, warmth or similar purposes.  2012 IFC, 202.

POLICY

The Fort Scott Fire Department has developed the following policy using the International Fire Code as adopted by the City of Fort Scott for the basis of the policy.

1.  Portable outdoor fireplaces shall not be operated within 15 feet of a structure, property line or combustible materials.

2.  Portable outdoor fireplaces shall not be used on a combustible deck.

3.  Recreational fires must be enclosed by some noncombustible fire ring at least 1 foot in height above the ground and shall not be constructed within 25 feet of a structure, property line or combustible materials.

4.  Outdoor fireplaces, other than portable, must be constructed of noncombustible material and shall not be constructed within 15 feet of a structure, property line or combustible materials unless constructed under the provisions of the International Residential Code.

5.  Outdoor fires that are offensive or objectionable because of smoke emissions or when atmospheric conditions or local circumstances make such fire hazardous shall be prohibited.  Outdoor fires shall not be permitted with the wind speeds exceed 15 MPH or when an area burn ban is in effect.

6.  Combustible materials which could cause a fire to spread shall be eliminated or at least mowed short in the immediate area prior to ignition.

7.  Ashes shall be discarded in a noncombustible container and kept outside of any structure.

8.  Only charcoal or clean dry seasoned firewood can be used for fuel.

9.  Do not use flammable and combustible fluids such as gasoline, alcohol, diesel fuel, kerosene or charcoal fluid to light and relight fires.

10.  Trash, garbage, leaves, asphalt, petroleum products, paint, rubber products, plastics, paper, cardboard, treated wood, construction debris, metal or any substance that normally releases toxic emissions, dense smoke, or obnoxious odors when burned shall not be used as a fuel.

11.  While a fire is burning or smoldering it must be attended at all times.

12.  A garden hose or water source shall be readily available at all times while a fire is burning or smoldering.

This policy is in effect for the City of Fort Scott.  If you have any questions concerning this policy, contact the Fort Scott Fire Department at (620) 223-2140.

10/4/19